Indy R2000 Module Series

Specification V1.1

- Product View

| Module Type | M–2600 | M-2800 | M-2900 |
| --- | --- | --- | --- |
| Real Photo |  |  |  |
| RF Channel | Single Channel | Four channels | Eight channels |
| RF Connector | MMCX | SMA | SMA |
| Antenna Connection Mode | Single Antenna | Four Antennas | Eight Antennas |
| Interface Connector | Molex 53261-1571 |  |  |
| RF Connectors Material | Gold-plated brass |  |  |
| PCB Material | Rogers FR4 gold-plated |  |  |
| Shield Material | Aluminum |  |  |

- PIN Connector Pin Assignments

| PIN ID | Interface | Instruction |
| --- | --- | --- |
| PIN 1 | GND | Meanwhile grounding |
| PIN 2 | GND |  |
| PIN 3 | 3.7V – 5V DC | Meanwhile connect power |
| PIN 4 | 3.7V – 5V DC |  |
| PIN 5 | GPIO 3 | Output |
| PIN 6 | GPIO 4 | Output |
| PIN 7 | GPIO 1 | Input |
| PIN 8 | Beeper | Has driven with > 50mA output current |
| PIN 9 | UART_RXD | TTL level |
| PIN 10 | UART_TXD |  |
| PIN 11 | USB_DM | For testing |
| PIN 12 | USB_DP |  |
| PIN 13 | GPIO 2 | Input |
| PIN 14 | EN | High level enable |
| PIN 15 | GPIO 5 | RS-485 direction control |

- Key Features

|  | Feature | Descriptions |
| --- | --- | --- |
| 1 | Impinj R2000 Built-in | * Impinj Indy R2000 chip as RF transceiver. |
| 2 | Anti-collision Algorithm | * Unique I - Search multi-tag identification algorithm providing the highest efficiency. |
| 3 | Optimized Algorithm for Tags with Small Volume | * Optimized applications for reading small volume tags with better respond time. |
| 4 | Dual CPU Architecture | * Main CPU: tag inventory; Assistant CPU: data management. * Tag inventory and data transfer are parallel and simultaneous. |
| 5 | Fast Antenna Switch Inventory (For M-2800 and M-2900) | * Every antenna’s inventory duration is configurable (Minimum duration: 30 ms). * Polling from ANT 1 to the last antenna. |
| 6 | Two Modes for Inventory | * Buffer mode and Real-time mode. * Tags will be stored as buffer under buffer mode. * Tags will send data under real-time mode. This mode allows user to get tag data instantly. |
| 7 | Hardware System Halt Detection | * Hardware CPU status surveillance. * Run for 24hours X 365 days without system halt. |
| 8 | PA Health Surveillance | * PA status surveillance. * Make sure PA never works under saturated state. Protect it for long term operation. |
| 9 | 18000-6B/6C Full Compatible | * It can be switched rapidly between 18000-6B and 18000-6C tag. |
| 10 | 18000-6B Large Data Read/Write | * Read 216 bytes in one time takes less than 500ms. * Write 216 bytes in one time takes less than 3.5 seconds. * It can read/write data with different lengths. |
| 11 | Antenna Connection Detection | * Detect antenna connection status. * Protective for RF receiver. * It can be canceled by command. |
| 12 | Temperature Sensor | * Multi-point surveillance for accurate operating system temperature. |
| 13 | Power Output Correction | * Dual modules making sure output power can be finely adjusted. * Dual modules working and keeping correction unless they are both damaged. |
| 14 | Excellent Cooling Design | * Heat dissipation and large cooling surface design. * Thermal coupling interfaces using high-thermal conductivity solid materials which ensure stable performance under high temperature. |

- Evaluation Kit

M-2600 Evaluation Kit

Figure 4-1: M-2600 Evaluation Kit front view

Figure 4-2: M-2600 Evaluation Kit side view

M-2800 Evaluation Kit

Figure 4-3: M-2800 Evaluation Kit front view

Figure 4-4: M-2800 Evaluation Kit side view

M-2900 Evaluation Kit

Figure 4-5: M-2900 Evaluation Kit front view

Figure 4-6: M-2900 Evaluation Kit side view

- Anti-Collision Algorithm Comparison

|  | Algorithm | Description |
| --- | --- | --- |
|  | Standard fixed Q algorithm | * Standard 18000-6C algorithm. * The performance is reduced significantly when tag quantity gets larger. * The efficiency is not high when tag quantity is small. |
|  | Impinj dynamic Q algorithm | * The algorithm of Impinj. * It has a good efficiency for various tag quantities. * It sacrifices some performance for the sake of compatibility. |
|  | I–Search dynamic Q algorithm V1.0 | * Based on Impinj dynamic Q algorithm. * The performance is optimized. * It’s the algorithm for firmware version 6.6 or below. |
|  | I–Search dynamic Q algorithm V2.0 | * Based on Impinj dynamic Q algorithm. * It’s a brand new data structure, the performance of which is significantly improved for firmware version 6.7 or above. * The improvement of performance can be easily sensed after the first round of inventory especially when the tag volume increases. |

Notes:

- ) The test is on same hardware platform in real applications (Taking Impinj dynamic Q algorithm as the reference which is marked with 100%).

- ) The chart shows the comparison for the first round inventory performance.

- ) It is tested on the same hardware platform.

- Electrical Characteristics

| Operating Voltage | 3.7V – 5 V |
| --- | --- |
| Standby Mode Current | <50mA (EN high level) |
| Sleep Mode Current | <100uA (EN low level) |
| Max Operating Current | 1.2A +/-10%, 600mA +/-10% (Low-power Mode, few tags) |
| Operating Temperature | - 20°C ~ +55°C |
| Storage Temperature | - 20°C ~ +85°C |
| Humidity | < 95% (+25°C) |
| Air Interface Protocol | EPC global UHF Class 1 Gen 2 / ISO 18000-6C / ISO 18000-6B |
| Spectrum Range | 860Mhz – 960Mhz |
| Supported Regions | US, Canada and other regions following U.S. FCC Europe and other regions following ETSI EN 302 208 Mainland China Japan Korea Malaysia Taiwan |
| Output Power | 0 – 33dBm |
| Output Power Precision | +/-1dB |
| Output Power Flatness | +/-0.2dB |
| Receive Sensitivity | < -85 dBm |
| Peak Inventory Speed | > 700 tags/sec |
| Tag Buffer Capacity | 1000 tags @ 96 bit EPC |
| Tag RSSI | Supported |
| Antenna Detector | Supported |
| Ambient Temp Monitor | Supported |
| Working Mode | Single/DRM |
| Host Communication | TTL Uart port |
| GPIO | 2 inputs & 2 outputs |
| Max Baud Rate | 115200 bps |
| Heat Dissipation | Air cooling |

Note:

◆ When the temperature, measured by the ambient temperature measurement function, exceeds 60°C, please do not keep the device working at full capacity.

◆ Please connect the device to heat sink when it continuously work at full load.

◆ Supply voltage must not exceed 5V, otherwise it will damage the internal protection circuit.

◆ Be cautious if set RF output power over 30dBm, as the peak current and internal temperature will increase significantly.

- Product Dimensions

M-2600 Structure Dimensions

Figure 7-1: Contour and hole location                  Figure 7-2: Mounting hole diameter

and shield location

Figure 7-3: Highest point and lowest point              Figure 7-4: Radiator mounting location

M-2800 Structure Dimensions

Figure 7-5: Contour and hole location                   Figure 7-6: Mounting hole diameter

and shield location

Figure 7-7: Space between highest point          Figure 7-8: Thickness between lowest point

and RF connector and PCB

Figure 7-9: Radiator mounting location

M-2900 Structure Dimensions

Figure 7-10: Contour and hole location             Figure 7-11: Mounting hole diameter

and shield location

Figure 7-12: Spacing between highest point          Figure 7-13: Thickness between lowest

and RF connector                                  point and PCB

Figure 7-14: Radiator mounting location

Note

◆ Figure 7-1 to 7-14 are three-dimensional renderings, non-physical.

◆ Any discrepancy, please defer to the real product instead.